Cognitive Robotics Project

What I have done so far.

- Completed all Tutorials – ran all examples successfully. Installed all libraries in my laptop using WSL and installed all libraries in the memory stick with Ubuntu. Both worked ok.

- Focus on PlanSys2 Examples

  - Explored PlanSys2_bt_example

  - Changed world and map.

  - Copy Turtle_Navigation/turtle_slam example into my WSL to use house_explored.yaml and house_explored.pgm.

  - Modify starting commands to:

- ros2 launch  turtlebot3_house_navigation patrol_example_launch.py  use_sim_time:=true

- ros2 launch nav2_bringup tb3_simulation_launch.py headless:=False use_sim_time:=true autostart:=True map:=/home/ozcascante/Turtle_Navigation/turtle_slam/maps/house_explored.yaml world:=/opt/ros/humble/share/turtlebot3_gazebo/worlds/turtlebot3_house.world

- Started Project using a simpler example. PlanSys2_patrol_navigation_example

  - Copy this example as starting point.

  - Change name to turtlebot3_house_navigation

    - Modified: CMakeLists.txt & package.xml

    - Keep /pddl/patrol.pddl unchanged.

    - Modify launch/patrol_example_launch.py. comment out nav2_cmd as it was causing Rviz to show the house map and the turtleworld.

    - Recorded coordinates for centers of all rooms.

    - Modify src/ move_action_node.cpp to include each house room coordinate and added a room6.

wp.pose.position.x = -2.4;

wp.pose.position.y = 2.3;

waypoints_["room1"] = wp;

wp.pose.position.x = -6.2;

wp.pose.position.y = 3.1;

waypoints_["room2"] = wp;

wp.pose.position.x = -6.3;

wp.pose.position.y = -1.6;

waypoints_["room3"] = wp;

wp.pose.position.x = 4.8;

wp.pose.position.y = 1.7;

waypoints_["room4"] = wp;

wp.pose.position.x = 6.2;

wp.pose.position.y = -1.9;

waypoints_["room5"] = wp;

wp.pose.position.x = 1.2;

wp.pose.position.y = 4.0;

waypoints_["room6"] = wp;

    - My laptop was struggling with the simulation, so I found changing some parameters in the param/nav2_params.yaml. Also decrease the Inflation Radius. (Robot struggling in corridor)

      - expected_planner_frequency: Change from 20.0 to  2.0. (The robot doesn't need to re-plan the long path every second).

      - controller_frequency: Change from 20.0 to 5.0. (This controls how often it calculates wheel speeds).

      - bt_loop_duration: increase it from 10 to  200.

      - Inflation Radius in costmap is too high, the robot thinks the doorway is blocked and give up. Lower inflation_radius to 0.2

- Created a commands.txt file. (made it executable). Commands to init the knowledge of the system and set the goal.

set instance leia robot

set instance room1 waypoint

set instance room2 waypoint

set instance room3 waypoint

set instance room4 waypoint

set instance room5 waypoint

set instance room6 waypoint

set predicate (connected room1 room2)

set predicate (connected room2 room1)

set predicate (connected room2 room3)

set predicate (connected room3 room2)

set predicate (connected room1 room4)

set predicate (connected room4 room1)

set predicate (connected room4 room5)

set predicate (connected room5 room4)

set predicate (connected room4 room6)

set predicate (connected room6 room4)

set predicate (robot_at leia room3)

set goal (and (robot_at leia room6))

- Run

- ros2 launch turtlebot3_gazebo turtlebot3_house.launch.py x_pose:=-6.3 y_pose:=-1.6

- ros2 launch turtlebot3_navigation2 navigation2.launch.py use_sim_time:=True map:=turtlebot3_house_navigation/maps/house_explored.yaml

  - Used 2D Pose to set the robot in RViz

- ros2 launch turtlebot3_house_navigation patrol_example_launch.py

- ros2 run plansys2_terminal plansys2_terminal

  - source commands.txt

  - run

  - It executes the plan and finds a way from room3 to room5 (bedroom2).

- Recording at https://youtu.be/JA05Y61rgAQ

- Created a small script to set the environment. Setup_tb3.sh. (as I have been typing this forever!!!)

#!/bin/bash

#source setup.bash

source install/setup.bash

# Set environment variables

export ROS_DOMAIN_ID=11

export TURTLEBOT3_MODEL=waffle

export GAZEBO_MODEL_PATH=$GAZEBO_MODEL_PATH:$(ros2 pkg prefix turtlebot3_gazebo)/share/turtlebot3_gazebo/models/

# Improves Gazebo performance

export LIBGL_ALWAYS_SOFTWARE=1

export SVGA_VGPU=0

# Print variables to verify

echo "ROS_DOMAIN_ID: $ROS_DOMAIN_ID"

echo "TURTLEBOT3_MODEL: $TURTLEBOT3_MODEL"

echo "GAZEBO_MODEL_PATH: $GAZEBO_MODEL_PATH"

echo "LIBGL_ALWAYS_SOFTWARE: $LIBGL_ALWAYS_SOFTWARE"

echo "SVGA_VGPU: $SVGA_VGPU"

- Run

ros2 launch turtlebot3_gazebo turtlebot3_house.launch.py x_pose:=-6.3 y_pose:=-1.6 use_sim_time:=True headless:=False autostart:=True

ros2 launch turtlebot3_navigation2 navigation2.launch.py use_sim_time:=True headless:=False autostart:=True map:=turtlebot3_house_navigation/maps/house_explored.yaml

- Used 2D Pose to set the robot in RViz

ros2 launch turtlebot3_house_navigation patrol_example_launch.py use_sim_time:=True

ros2 run plansys2_terminal plansys2_terminal - source commands.txt - run